DL壳

DC线输出IT/家电/LED控制装置/医疗电源系列

特点 Features:

100-240VAC Universal Input

Single Output to 12W Max

Regulated Output with Low Ripple Noise

机械细节 Mechanical Details

| Case Dimensions | L52.5*W39*H25mm |
| --- | --- |
| 欧规、美规、英规、澳规、日规、中规固定AC插脚和可换脚可选 |  |

输入输出/Input & Output

| Model | I/P | O/P |  |  |
| --- | --- | --- | --- | --- |
|  |  | Voltage | Current | Watt |
| HNDLxxxyyywv | Voltage：100-240 VAC Line Frequency：50/60Hz Current：0.45A Max. | 45-24.0V | 0.01-2.0A | 12W(Max) |

保护 Protection

| Overload | 110-200% rated output power,auto recovery(hiccup mode) |
| --- | --- |
| Over Voltage | 150-200% rated output Voltage,auto recovery(hiccup mode) |
| Short Circuit | auto recovery(hiccup mode) |

安规认证 / Safety Approvals

UL62368-1;UL1310;FCC Part 15 Subpart B Class B；
IEC/EN62368-1;IEC/EN61558-2-16;IEC/EN60335-2-29;

IEC/EN61347-1&IEC/EN61347-2-13;UL8750;

UL/IEC60601-1;IEC60601-1-2;

EN55032;EN55014-1;EN55035;EN55014-2;
GB4943.1-2011;GB/T9254-2008;GB17625.1-2012；

GB4706.1-2005;GB/T2099.1-2008；

GB19510.1&GB19510.14;

J60950-1(H27);J55022(H22)；

J61558-1(H26)&J61558-2-16(H26);J55014-1(H27);
J60335-1(H27) J60335-2-29(H27) J55014-1(H27)；

AS/NZS60950.1:2015;CIS PR32；

AS/NZS61558.1&AS/NZS61558.2.16;CISPR 14；

AS/NZS60335.1&AS/NZS60335.2.29;CISPR 14；

CB CE GS ETL FCC CCC CQC PSE SAA RCM VI

电气性能 Electrical

| Topology | Switching Flyback |
| --- | --- |
| Insulation resistance | Input ot output: 10MΩmin. At 500VDC |
| Dielectric Withstand | 3000Vac/60Sec @ Input to Output |
| Leakage Current | 0.1mA @ 2Pin Input |
| Ripple | 150mV Max |
| Transient Response | 0.5mS for 50%Load Change Typical |
| Holdup Time | 6mS Min @ 100Vac/60Hz Max load |
| Stand by power | 0.1W without load |
| MTBF | ＞35，000 hrs MIL-HDBK-217 at 25℃ |
| USB or DC Cord | DC Cord |

环境 Environmental

| Operaturing Temperature | 0℃ to 35℃ |
| --- | --- |
| Storage Temperature | -20℃ to 60℃ |
| Relative Humidity | Operating:20% to 80% RH |
|  | Storage:10% to 90% RH |
| CoolingNatural | Yes |