SE壳

DC线输出IT/AV/家电电源系列

特点 Features:

100-240VAC Universal Input

Single Output to 24W Max

Regulated Output with Low Ripple Noise

机械细节 Mechanical Details

| Case Dimensions | L70*W47.5*H30mm |
| --- | --- |
| 欧规、美规、英规固定AC插脚和可换脚可选 |  |

输入输出/Input & Output

| Model | I/P | O/P |  |  |
| --- | --- | --- | --- | --- |
|  |  | Voltage | Current | Watt |
| HNSExxxyyyWv | Voltage：100-240 VAC Line Frequency：50/60Hz Current：0.6A Max. | 4.5-7.5V | 0.01-4.0A | 24W(Max) |
|  |  | 9.0-16.0V | 0.01-2.5A |  |
|  |  | 16.1-24.0V | 0.01-1.4A |  |

保护 Protection

| Overload | 110-200% rated output power,auto recovery(hiccup mode) |
| --- | --- |
| Over Voltage | 150-200% rated output Voltage,auto recovery(hiccup mode) |
| Short Circuit | auto recovery(hiccup mode) |

安规认证 / Safety Approvals

UL62368-1;FCCPart 15 Subpart B Class B

IEC/EN62368-1;EN55032;EN55035;

GB4943.1-2011;GB/T9254-2008;GB17625.1-2012

GB4706.1-2005;GB/T2099.1-2008；

J60950-1(H27);J55022(H22)

AS/NZS60950.1:2015;CIS PR32

K60950-1;KN22 KN24

CB CE GS ETL FCC CCC CQC PSE SAA RCM KC VI

电气性能 Electrical

| Topology | Switching Flyback |
| --- | --- |
| Insulation resistance | Input ot output: 10MΩmin. At 500VDC |
| Dielectric Withstand | 3000Vac/60Sec @ Input to Output |
| Leakage Current | 0.25mA @ 2Pin Input |
| Ripple | 150mV Max |
| Transient Response | 0.5mS for 50%Load Change Typical |
| Holdup Time | 6mS Min @ 100Vac/60Hz Max load |
| Stand by power | 0.1W without load |
| MTBF | ＞35，000 hrs MIL-HDBK-217 at 25℃ |
| USB or DC Cord | DC Cord |

环境 Environmental

| Operaturing Temperature | 0℃ to 35℃ |
| --- | --- |
| Storage Temperature | -20℃ to 60℃ |
| Relative Humidity | Operating:20% to 80% RH |
|  | Storage:10% to 90% RH |
| CoolingNatural | Yes |